LAB 2

# SCT212-0204/2021

# OMONDI KRISS

Problem

Use the following code fragment:

loop: LD R1,0(R2) DADDI R1,R1,1

SD 0(R2),R1

DADDI R2,R2,4  DSUB R4,R3,R2 BNEZ R4,loop

Assume that the initial value of R3 is R2+396. Throughout this exercise use the classic RISC five- stage integer pipeline in H&P. Specifically, assume that:

1)   branches are resolved in the second stage of the pipeline;

2)  there are separate instruction and data memories;

3)   all memory accesses take 1 clock cycle.

a. Show the timing of this instruction sequence for the RISC pipeline without any forwarding or   bypassing hardware but assuming a register read and a write in the same clock cycle “forwards” through the register file. Assume that the branch is handled by flushing the pipeline. If all

memory references take 1 cycle, how many cycles does this loop take to execute?

| Clock Cycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1: LD R1 | IF | ID | EX | MEM | WB |  |  |  |  |  |  |  |
| 2: DADDI |  | IF | ID | stall | stall | EX | MEM | WB |  |  |  |  |
| 3: SD |  |  | IF | ID | stall | stall | stall | stall | EX | ME | WB |  |
| 4: DADDI |  |  |  | IF | ID | EX | ME | WB |  |  |  |  |
| 5: DSUB |  |  |  |  | IF | ID | stall | stall | EX | ME | WB |  |
| 6: BNEZ |  |  |  |  |  | IF | ID | stall | stall | stall | stall | EX |
| 7: Next Seq |  |  |  |  |  |  | IF | stall | stall | stall | stall | Flush |

Cycles per Loop Iteration: 12 cycles

b. Show the timing of this instruction sequence for the RISC pipeline with normal forwarding    and bypassing hardware. Assume that the branch is handled by predicting it as not taken. If all memory references take 1 cycle, how many cycles does this loop take to execute?

Full Forwarding: Results from EX/MEM stages are available to be forwarded to the EX/ID stages of subsequent instructions to minimize stalls.

Load-Use Hazard: A 1-cycle stall still occurs if an instruction needs the result of a LD in the immediately following cycle

Predict-Not-Taken: The pipeline assumes the branch (BNEZ) will not be taken and fetches the next sequential instruction

| Clock Cycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1: LD | IF | ID | EX | ME | WB |  |  |  |
| 2: DADDI |  | IF | ID | stall | EX | ME | WB |  |
| 3:SD |  |  | IF | ID | EX | ME | WB |  |
| 4: DADDI |  |  |  | IF | ID | EX | ME | WB |
| 5: DSUB |  |  |  |  | IF | ID | EX | ME |
| 6: BNEZ |  |  |  |  |  | IF | ID | EX |
| 7: Next Seq |  |  |  |  |  |  | IF | Flush |

Cycles per Loop Iteration: 8 cycles

c. Assume the RISC pipeline with a single-cycle delayed branch and normal forwarding and

bypassing hardware. Schedule the instructions in the loop including the branch delay slot. You  may reorder instructions and modify the individual instructions operands, but do not undertake other loop transformations that change the number or opcode of the instructions in the loops.   Show a pipeline timing diagram and compute the number of cycles needed to execute the entire loop.

Scheduled Code

loop: LD     R1, 0(R2)

DADDI  R1, R1, 1   DSUB   R4, R3, R2 SD     0(R2),

BNEZ   R4, loop   DADDI  R2, R2, 4

| Clock Cycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1: LD | IF | ID | EX | ME | WB |  |  |
| 2: DADDI |  | IF | ID | stall | EX | ME | WB |
| 5: DSUB |  |  | IF | ID | EX | ME | WB |
| 3: SD R1 |  |  |  | IF | ID | EX | ME |
| 6: BNEZ |  |  |  |  | IF | ID | EX |
| 4: DADDI |  |  |  |  |  | IF | ID |
| 7: LD (loop) |  |  |  |  |  | IF |  |